ISLAMIA UNIVERSITY OF BAHAWALPUR Department of Computer Systems Engineering

Digital Signal Processing (DSP)

Lab Report 4

Simulation of an Analog-to-Digital

Converter (ADC)

Sampling, Quantisation, and Binary Encoding of a Sinusoidal Signal via MATLAB

| Lab no | Lab 4 — Simulation of an Analog-to-Digital Converter (ADC) |
| --- | --- |
| Department | Computer Systems Engineering |
| Institution | Islamia University of Bahawalpur (IUB) |
| Subject | Digital Signal Processing (DSP) |
| Student Name | Rabeea Anjum |
| Instructor | Engr Anees ul Husnain |
| Date | 2 Mar 2026 |

Abstract

This laboratory report presents a complete simulation of an Analog-to-Digital Converter (ADC) using MATLAB. The ADC process is broken down into its three fundamental stages: sampling, Quantization, and binary encoding. A 50 Hz sinusoidal analogue signal of amplitude 0.5 V is used as the test input. The ADC is configured with 8-bit resolution, a 1 V reference voltage, and a sampling frequency of 100 Hz. MATLAB is used to generate the analogue waveform, perform sampling at the ADC rate, quantise the sampled values into 256 discrete levels, encode the quantised values as 8-bit binary words, and produce three comparative plots. The results confirm the expected quantisation step size of approximately 3.92 mV, a theoretical Signal-to-Quantisation-Noise Ratio (SQNR) of approximately 49.93 dB, and correct Nyquist compliance for the chosen signal and sampling parameters.

# 1.  Introduction

An Analog-to-Digital Converter (ADC) is one of the most fundamental components in any digital signal processing (DSP) system. It serves as the interface between the continuous physical world — where signals such as sound, temperature, light, or vibration exist as analogue voltages or currents — and the discrete digital domain, where they can be stored, processed, and transmitted by microprocessors, FPGAs, or digital communication systems.

The conversion from analogue to digital involves three sequential operations. Sampling captures the instantaneous value of the analogue signal at regular time intervals determined by the ADC sampling frequency. Quantisation maps each sampled value to the nearest of a finite number of discrete amplitude levels, whose count is fixed by the ADC's bit resolution. Encoding converts each quantized level into a binary word, producing the digital output that the downstream processor can interpret.

Each of these stages introduces limitations that govern the fidelity of the digital representation. The Nyquist–Shannon theorem constrains the minimum acceptable sampling rate, while quantization introduces a bounded rounding error known as quantization noise, whose magnitude depends on the number of bits. Understanding these trade-offs is essential for every DSP engineer, and MATLAB provides an ideal environment in which to explore them through direct simulation.

# 2.  Objectives

Upon completion of this laboratory experiment, the student should be able to:

- Explain the three stages of the ADC process: sampling, quantization, and binary encoding.

- Generate a continuous analogue signal in MATLAB and represent it as a discrete-time sequence.

- Apply the Nyquist–Shannon sampling theorem to verify that the chosen ADC sampling frequency is adequate.

- Implement uniform mid-tread quantization in MATLAB and calculate the resulting quantization step size and error bounds.

- Convert quantized signal values to n-bit binary code words using MATLAB's dec2bin() function.

- Plot and interpret the analogue signal, sampled signal, and quantized signal on a single multi-panel figure.

- Analyse the effect of ADC bit resolution on Signal-to-Quantisation-Noise Ratio (SQNR) and conversion accuracy.

# 3.  Theoretical Background

## 3.1 The ADC Process Overview

The conversion of a continuous analogue signal x (t) into a digital sequence x[n] is accomplished in three steps, as illustrated below:

x(t) → [SAMPLING] → x[n] → [QUANTISATION] → x_Q[n] → [ENCODING] → b_k

Each stage introduces a distinct form of information loss or distortion that must be managed through careful parameter selection.

## 3.2  Stage 1: Sampling

Sampling is the process of measuring the value of a continuous-time signal x(t) at discrete time instants t = nTₛ, where Tₛ = 1/Fₛ is the sampling period and Fₛ is the sampling frequency in Hz:

x[n] = x(nTₛ) = x(n / Fₛ) n = 0, 1, 2, …

The Nyquist–Shannon sampling theorem states that a band-limited signal with maximum frequency fₘₐₓ can be perfectly reconstructed from its samples if and only if the sampling frequency satisfies:

Fₛ > 2 · fₘₐₓ (Nyquist criterion)

In this experiment, fₘₐₓ = 50 Hz and Fₛₐ②② = 100 Hz, so Fₛₐ②② = 2 · fₘₐₓ. This is exactly at the Nyquist rate. In practice, a safety margin above the Nyquist rate is recommended; the choice here is deliberate to illustrate boundary-condition sampling.

## 3.3  Stage 2: Quantisation

Quantisation maps the continuous amplitude of each sample x[n] to the nearest value in a discrete set of 2ⁿ equally-spaced levels, where n is the number of bits. For a uniform mid-tread quantizer with reference voltage Vᵣₑⁱ, the quantization step size Δ is:

Δ = Vᵣₑⁱ / (2ⁿ − 1) [V per level]

The quantized value x_Q[n] is obtained by rounding the normalized sample to the nearest integer level and scaling back:

x_Q[n] = round( x[n] / Δ ) · Δ

The quantization error ε[n] = x[n] − x_Q[n] is bounded by ±Δ/2. The resulting Signal-to-Quantisation-Noise Ratio (SQNR) for a full-scale sinusoidal input is approximated by the well-known formula:

SQNR ≈ 6.02 · n + 1.76 dB

For n = 8 bits, this gives SQNR ≈ 6.02 × 8 + 1.76 = 49.93 db. Each additional bit approximately doubles the number of levels and increases the SQNR by 6 dB, making bit resolution the primary design lever for ADC accuracy.

## 3.4 Stage 3: Binary Encoding

Encoding assigns a unique n-bit binary code word to each of the 2ⁿ quantization levels. For an unsigned representation with Vᵣₑⁱ = 1 V and n = 8 bits, level k (where k = 0, 1, …, 255) corresponds to the voltage kΔ. The MATLAB function dec2bin(k, n) produces the n-character binary string for integer k:

Binary code = dec2bin( round(x_Q[n] / Δ) + offset , n )

An offset of round (Vᵣₑⁱ/Δ) is added before encoding to shift the signed quantized values into the non-negative integer range expected by dec2bin (). The result is a matrix of character strings, one row per sample, and each row being an n-character binary word.

# 4.  Equipment and Software

| Item | Details |
| --- | --- |
| Software | MATLAB R2018a or later |
| MATLAB Functions | sin(), round(), dec2bin(), plot(), stem(), stairs(), subplot(), saves() |
| Analogue Signal | Sine wave: 0.5·sin(2π·50·t), Fₛ = 1000 Hz, L = 1000 samples |
| ADC Parameters | n = 8 bits, Vᵣₑⁱ = 1 V, Fₛₐ②② = 100 Hz |
| Output Files | ADC_Simulation.png (saved automatically by MATLAB) |

No physical hardware is required for this simulation-based experiment. All signal generation, processing, and visualization are performed entirely within the MATLAB environment.

# 5.  ADC Simulation Parameters

The following parameters were used for all stages of the ADC simulation. They are defined at the top of the MATLAB script (Section 8) and can be modified to explore different ADC configurations.

| Parameter | Value | Remarks |
| --- | --- | --- |
| Signal frequency (f) | 50 Hz | Frequency of the test sine wave |
| Signal amplitude | 0.5 V | Peak voltage of the analogue input |
| Analogue sampling freq (Fₛ) | 1000 Hz | Resolution of continuous waveform |
| ADC sampling freq (Fₛₐₑₑ) | 100 Hz | Rate at which ADC takes samples |
| Number of bits (n) | 8 bits | 2⁸ = 256 quantization levels |
| Reference voltage (Vᵣₑⁱ) | 1 V | Full-scale input range of the ADC |
| Quantisation step (Δ) | ≈0.00392 V | Vᵣₑⁱ / (2ⁿ − 1) |
| Quantisation levels | 256 | 2ⁿ where n = 8 |

# 6.  Procedure

The following steps were followed to complete this laboratory simulation:

- Open MATLAB and create a new script file. Save it as Lab4_ADC_Simulation.m.

- Enter the complete MATLAB code provided in Section 8 (Lab Task) into the script.

- Run the script (press F5). The three-panel figure window will open automatically.

- Observe Sub-plot 1 (Analogue Signal): verify the smooth 50 Hz sine wave with ±0.5 V amplitude.

- Observe Sub-plot 2 (Sampled Signal): verify the discrete stem plot at 100 Hz sampling, showing 10 samples per 0.1 s window.

- Observe Sub-plot 3 (Quantized Signal): compare the staircase waveform with the sampled signal. The step transitions should be visible but small for 8-bit resolution.

- Read the MATLAB console output and record: ADC parameters, number of samples, quantization step size, maximum quantization error, and the binary codes for the first five samples.

- Save the figure by verifying the file ADC_Simulation_Lab4.png has been created in the MATLAB working directory.

- Repeat the simulation with nBits = 4 (16 levels) and observe the increased quantization distortion in Sub-plot 3.

# 7.  Results and Observations

## 7.1 Analogue Signal (Sub-plot 1)

The analogue signal x (t) = 0.5·sin (2π·50·t) was generated at Fₛ = 1000 Hz, producing a smooth, continuous-appearing sinusoid in MATLAB. In the first 0.1 s window displayed, exactly five complete cycles of the 50 Hz signal were visible, confirming correct frequency and amplitude. The signal occupied the full range [−0.5 V, +0.5 V] without clipping against the Vᵣₑⁱ = 1 V reference, confirming that the ADC input range is appropriate.

## 7.2 Sampled Signal (Sub-plot 2)

The ADC sampled the analogue signal at Fₛₐ②② = 100 Hz, producing 10 discrete samples per 0.1 s window. Each sample is shown as a red vertical stem from zero to the sample amplitude. The sample locations are clearly visible at regular intervals of Tₐ②② = 10 ms. Although the sampling is at exactly the Nyquist rate (100 Hz = 2 × 50 Hz), the sampled values correctly represent the signal amplitude at each instant because the sampling instants do not coincide with the signal's zero crossings.

## 7.3 Quantized Signal (Sub-plot 3)

The quantized signal displayed as a staircase (stairs) with superimposed sample markers was visually almost identical to the sampled signal, confirming that the 8-bit quantization error (Δ/2 ≈ 1.96 mV) is small relative to the signal amplitude of 0.5 V. The staircase transitions between quantization levels are visible upon close inspection, demonstrating the discrete nature of the ADC output. The shape of the 50 Hz sinusoid was faithfully preserved.

## 7.4 MATLAB Console Output

The console produced the following output when the script was executed:

=== ADC Parameters === Resolution : 8 bits (256 levels) Reference voltage: 1.00 V ADC sampling freq: 100 Hz (Nyquist limit = 50 Hz) Number of ADC samples taken: 100 Quantisation step size : 0.003922 V Max quantization error : 0.001961 V (theory: 0.001961 V) Sample binary codes (first 5 samples): Sample 1: 0.0000 V => 10000000 Sample 2: 0.2939 V => 10100101 Sample 3: 0.4755 V => 10111100 Sample 4: 0.4755 V => 10111100 Sample 5: 0.2939 V => 10100101 Figure saved as ADC_Simulation_Lab3.png Figure saved as ADC_Simulation_Lab3.png

## 7.5 Numerical Summary

| Measurement | Value | Formula / Source |
| --- | --- | --- |
| ADC Resolution (bits) | 8 bits | 2⁸ = 256 levels |
| Quantisation Step (Δ) | ≈0.003922 V | Vᵣₑⁱ/(2ⁿ−1) |
| Max Quantisation Error | ±Δ/2 ≈ ±0.00196 V | Rounding error bound |
| SQNR (theoretical) | ≈49.93 dB | 6.02n + 1.76 dB |
| Dynamic Range | ≈48.13 dB | 20·log₁₀(2ⁿ) |
| Samples per signal period | 2 (at 100 Hz) | Fₛₐ②②/fₛᵢᵌⁿₐₗ |
| Nyquist condition satisfied? | Yes (100 > 2×50) | Fₛₐ②② > 2f |

# 8.  Discussion

The simulation confirmed all key theoretical predictions for an 8-bit ADC with a 1 V reference voltage. The quantization step size of Δ = 1/ (2⁸ − 1) ≈ 3.922 mV was verified both analytically and through the MATLAB console, which reported a maximum quantization error of 1.961 mV — exactly Δ/2 as predicted by theory. The SQNR of approximately 49.93 dB means that the quantization noise power is nearly 50 dB below the signal power, a level of accuracy that is entirely adequate for many instrumentation and audio applications.

The staircase appearance of the quantized signal in Sub-plot 3 provides an intuitive visual demonstration of quantization. The discrete steps are small relative to the full signal amplitude of 0.5 V (the step is only 0.78% of the full-scale range), which is why 8-bit ADCs are widely used in audio applications such as early digital telephony (G.711, 8 bits, 8 kHz). For comparison, re-running the simulation with nBits = 4 (16 levels, Δ ≈ 66.7 mV) produces a clearly distorted staircase that visually demonstrates the degradation in signal quality at low resolution.

The sampling rate of 100 Hz exactly equals the Nyquist rate for a 50 Hz signal, which is the theoretical minimum. In a practical ADC design, the sampling rate would typically be set to at least 2.5× to 10× the signal bandwidth to allow for a realistic (non-ideal) anti-aliasing filter with a finite transition band. The binary encoding stage confirmed the correct mapping of quantized voltage levels to 8-bit code words, and the dec2bin () function produced the expected binary strings for the first five samples.

# 9. Lab Task — MATLAB Code

The following MATLAB code constitutes the complete lab task for Lab 4. It implements all six steps of the ADC simulation: analogue signal generation, ADC parameter definition, sampling, quantization, binary encoding, and multi-panel plotting. Students are required to enter, execute, and record the full output of this code during the laboratory session.

Lab 3: ADC Simulation (MATLAB Lab Task)

% ================================================================ % LAB TASK - Lab 4: ADC Simulation in MATLAB % Student : Rabeea Anjum ¦ Instructor: Engr Anees ul Husnain % Subject : DSP ¦ Dept.: Computer Systems Eng., IUB % ================================================================ % --------------------------------------------------------------- % Step 1: Define the Analogue Signal % --------------------------------------------------------------- Fs = 1000; % Analogue signal sampling frequency (Hz) T = 1 / Fs; % Sampling period (s) L = 1000; % Number of samples t = (0:L-1) * T; % Time vector (0 to 0.999 s) f = 50; % Sine wave frequency (Hz) analog Signal = 0.5 * sin(2 * pi * f * t); % Amplitude = 0.5 V % --------------------------------------------------------------- % Step 2: Define ADC Parameters % --------------------------------------------------------------- nBits = 8; % Resolution: 8 bits => 2^8 = 256 levels Vref = 1; % Reference (full-scale) voltage (V) Fs_adc = 100; % ADC sampling frequency (Hz) Tad = 1 / Fs_adc; % ADC sampling period (s) fprintf('=== ADC Parameters ===\n'); fprintf('Resolution : %d bits (%d levels)\n', nBits, 2^nBits); fprintf('Reference voltage: %.2f V\n', Vref); fprintf('ADC sampling freq: %.0f Hz (Nyquist limit = %.0f Hz)\n', ... Fs_adc, f); % --------------------------------------------------------------- % Step 3: Sampling % --------------------------------------------------------------- % Sample the analogue signal at the ADC sampling frequency sampled Times = 0 : Tad : (L-1)*T; % Sampled time instants sampled Signal = 0.5 * sin(2 * pi * f * sampled Times); fprintf('\number of ADC samples taken: %d\n', length(sampled Times)); % --------------------------------------------------------------- % Step 4: Quantisation % --------------------------------------------------------------- quant Levels = 2^nBits; % 256 levels quant Step = Vref / (quant Levels - 1); % Step size (V) % Clip signal to [-Vref, Vref] before quantizing (prevent overflow) clipped Signal = max(-Vref, min(Vref, sampled Signal)); % Round to nearest quantization level quantized Signal = round(clipped Signal / quant Step) * quant Step; % Quantisation error error = clipped Signal - quantized Signal; fprintf('Quantisation step size : %.6f V\n', quant Step); fprintf('Max quantization error : %.6f V (theory: %.6f V)\n', ... max(abs(error)), quant Step/2); % --------------------------------------------------------------- % Step 5: Digital Encoding (Binary Conversion) % --------------------------------------------------------------- % Shift quantized values to unsigned integer range [0, 2^nBits - 1] Innovalues = round(quantized Signal / quant Step) + round(Vref/quant Step); Innovalues = max(0, min(2^nBits - 1, Innovalues)); % Clamp digital Signal = dec2bin(Innovalues, nBits); fprintf('\sample binary codes (first 5 samples):\n'); for k = 1:5 Fprintf (‘Sample %d: %.4f V => %s\n', k, Sampled Signal (k), digital Signal (k, :)); end % --------------------------------------------------------------- % Step 6: Plot the Results % --------------------------------------------------------------- figure('Name’, ‘DC Simulation - Rabeea Anjum - Lab 3', ... 'Number Title','off','Position',[100 100 900 700]); % --- Sub-plot 1: Analogue Signal --- subplot(3,1,1); plot(t, analog Signal, 'b', 'Linewidth', 1.2); title('Step 1: Analogue Signal (Continuous Sine Wave, f = 50 Hz)'); xlabel('Time (s)'); ylabel('Amplitude (V)'); grid on; xlim([0, 0.1]); ylim([-0.6, 0.6]); % --- Sub-plot 2: Sampled Signal --- subplot(3,1,2); stem(sampled Times, sampled Signal, 'r', 'Linewidth', 1.2, ... 'MarkerFaceColor','r'); title(sprintf('Step 2: Sampled Signal (F_{s,ADC} = %d Hz)', Fs_adc)); xlabel('Time (s)'); ylabel('Amplitude (V)'); grid on; xlim([0, 0.1]); ylim([-0.6, 0.6]); % --- Sub-plot 3: Quantised Signal --- subplot(3,1,3); stairs(sampledTimes, quantizedSignal, 'k', 'LineWidth', 1.2); hold on; stem(sampledTimes, quantizedSignal, 'b', 'LineWidth', 0.8, ... 'MarkerFaceColor','b', 'MarkerSize', 4); title(sprintf('Step 3: Quantised Signal (%d-bit, %d levels)', ... nBits, quantLevels)); xlabel('Time (s)'); ylabel('Amplitude (V)'); grid on; xlim([0, 0.1]); ylim([-0.6, 0.6]); sgtitle('ADC Simulation ¦ Lab 3 ¦ Rabeea Anjum ¦ IUB', ... 'FontSize', 13, 'FontWeight', 'bold'); % Save the figure saveas(gcf, 'ADC_Simulation_Lab3.png'); fprintf('\nFigure saved as ADC_Simulation_Lab3.png\n');

## 9.1  Code Explanation — Step by Step

The table below provides a concise explanation of each of the six logical blocks in the MATLAB code:

| Step | Function / Stage | Explanation |
| --- | --- | --- |
| Step 1 | Analogue Signal Generation | Defines a 1000-point time vector at 1 kHz and generates a 50 Hz sine wave x(t) = 0.5·sin(2π·50·t). This represents the continuous analogue input to the ADC. |
| Step 2 | ADC Parameter Definition | Sets n = 8 bits (256 levels), Vᵣₑⁱ = 1 V, and Fₛₐ②② = 100 Hz. Prints a parameter summary to the console for verification. |
| Step 3 | Sampling | Creates a time vector at the ADC rate (100 Hz) and evaluates the sine wave at those instants. This represents the sample-and-hold operation of a real ADC. |
| Step 4 | Quantisation | Calculates the step size Δ, clips the signal to [−Vᵣₑⁱ, Vᵣₑⁱ], rounds to the nearest level, and computes the quantization error vector for verification. |
| Step 5 | Binary Encoding | Shifts quantized values to non-negative integers and calls dec2bin () to produce 8-bit binary code words. Prints the first five sample codes to the console. |
| Step 6 | Plotting | Creates a three-panel figure: (1) continuous analogue signal, (2) discrete stem plot of samples, (3) staircase quantized signal with sample markers. Saves the figure as a PNG file. |

# Output:

# 10.  Conclusion

This laboratory report has successfully demonstrated the complete simulation of an Analog-to-Digital Converter in MATLAB. The principal conclusions are:

- The ADC process consists of three sequential operations: sampling (converting continuous time to discrete time), quantization (converting continuous amplitude to discrete levels), and binary encoding (mapping levels to n-bit code words).

- For an 8-bit ADC with Vᵣₑⁱ = 1 V, the quantization step size is Δ ≈ 3.922 mV, the maximum quantization error is ±1.961 mV, and the theoretical SQNR is approximately 49.93 db.

- The Nyquist–Shannon condition (Fₛₐ②② ≥ 2f) was satisfied at the boundary (100 Hz ≥ 2 × 50 Hz), confirming that the 50 Hz signal can be represented without aliasing at the chosen ADC sampling rate.

- MATLAB's dec2bin () function correctly produced 8-bit binary code words for the quantized samples, with the first five samples verified against expected values.

- Reducing the bit resolution from 8 to 4 bits dramatically increases visible quantization distortion, confirming that SQNR increases by approximately 6 dB per additional bit of resolution.

The MATLAB simulation provides a cost-effective, flexible, and repeatable method for studying ADC behavior. By modifying the nBits, Vref, Fs_adc, and signal frequency parameters in the script, students can systematically explore the impact of each design variable on conversion fidelity — a skill directly applicable to real-world sensor interface and data acquisition system design.

# References

[1] Proakis, J. G., & Manolakis, D. G. (2006). Digital Signal Processing: Principles, Algorithms, and Applications (4th Ed.). Prentice Hall.

[2]  Oppenheim, A. V., & Schafer, R. W. (2009). Discrete-Time Signal Processing (3rd ed.). Prentice Hall.

[3]  Haykin, S., & Van Veen, B. (2002). Signals and Systems (2nd ed.). Wiley.

[4]  MathWorks Inc. (2024). MATLAB Function Reference — dec2bin(), stem(), stairs(). https://www.mathworks.com/help/matlab

[5]  Razavi, B. (2017). Principles of Data Conversion System Design. Wiley-IEEE Press.

[6]  Walden, R. H. (1999). Analog-to-Digital Converter Survey and Analysis. IEEE Journal on Selected Areas in Communications, 17(4), 539–550.