ISLAMIA UNIVERSITY OF BAHAWALPUR Department of Computer Systems Engineering

Digital Signal Processing (DSP)

Lab Report 5

The Importance of Bode Plots in

Digital Signal Processing

Frequency Response Analysis, Gain & Phase — with MATLAB Implementation

| Lab no | Lab 5 — Bode Plots in Digital Signal Processing |
| --- | --- |
| Department | Computer Systems Engineering |
| Institution | Islamia University of Bahawalpur (IUB) |
| Subject | Digital Signal Processing (DSP) |
| Student Name | Rabeea Anjum |
| Instructor | Engr Anees ul Husnain |
| Date | 9 Mar 2026 |

Abstract

This report investigates the theory and practical application of Bode plots within the domain of Digital Signal Processing (DSP). Bode plots are fundamental frequency-domain tools that simultaneously display a system's magnitude response (in dB) and phase response (in degrees) across a logarithmic frequency axis. The report covers the mathematical basis of frequency response, gain, and phase for Linear Time-Invariant (LTI) systems, elaborates on the engineering significance of Bode plots in filter design and stability analysis, and provides a complete MATLAB implementation as the lab task. Experimental results confirm the theoretical characteristics of a first-order low-pass filter, validating both the analytical model and the MATLAB workflow.

# 1.  Introduction

In Digital Signal Processing (DSP), characterizing the behavior of a system across a range of input frequencies is a fundamental requirement. Whether designing a digital filter, evaluating a control loop, or analysing a communication channel, engineers need to know how much a system amplifies or attenuates a signal at each frequency, and by how much it shifts the signal in time.

The Bode plot is one of the most powerful and widely-used tools for this purpose. Developed by Hendrik Wade Bode at Bell Laboratories in the 1930s, it remains a cornerstone technique in both classical control theory and modern DSP. A Bode plot presents a system's frequency response as two separate panels:

- Magnitude Plot — displays the system gain in decibels (dB) as a function of angular frequency ω (rad/s) on a logarithmic scale.

- Phase Plot — displays the phase shift (in degrees) introduced by the system as a function of angular frequency ω on the same logarithmic scale.

In this lab, MATLAB's Control System Toolbox is used to construct, plot, and numerically analyse the Bode response of a first-order low-pass filter, illustrating all key theoretical concepts in a practical setting.

# 2.  Objectives

Upon completion of this laboratory experiment, the student should be able to:

- Define the concept of frequency response for an LTI system and explain the terms gain and phase shift.

- Interpret the two components of a Bode plot and relate them to the system transfer function.

- Identify the cutoff frequency, roll-off rate, and phase margin directly from a Bode plot.

- Use MATLAB's tf() and bode() functions to model a transfer function and generate its Bode plot.

- Verify analytical predictions numerically by extracting magnitude and phase values at specific frequencies.

# 3.  Theoretical Background

## 3.1  Frequency Response of an LTI System

For a Linear Time-Invariant (LTI) system described by the transfer function H(s), the frequency response is obtained by substituting s = jω, where j = √(−1) and ω is the angular frequency in rad/s. The resulting complex-valued function H(jω) encodes both the gain and phase behaviour of the system:

H(jω) = ¦H(jω)¦ · e^{ j · arg[H(jω)] }

This expression contains two real-valued functions of frequency: the magnitude |H(jω)| and the phase angle arg[H(jω)], plotted separately in the two panels of a Bode diagram.

## 3.2  Gain (Magnitude)

The gain at a given frequency ω describes how much the system scales the amplitude of a sinusoidal input at that frequency. It is defined as the ratio of output amplitude to input amplitude:

Gain = ¦H(jω)¦ = Output Amplitude / Input Amplitude

In a Bode magnitude plot, gain is expressed in decibels (dB) to compress the dynamic range and linearise multiplicative relationships:

Gain (dB) = 20 · log₁₀ ¦H(jω)¦

A gain of 0 dB means no amplification or attenuation. For a first-order low-pass filter the gain is 0 dB at low frequencies and rolls off at −20 dB per decade above the cutoff frequency.

## 3.3  Phase

The phase describes the angular delay imposed on a sinusoidal input by the system. It is the argument of the complex number H(jω):

Phase = ∠H(jω) = arg[ H(jω) ] (degrees or radians)

For H(s) = 1/(s+1), the phase response is φ(ω) = −arctan(ω/ωc). The phase starts at 0° for ω → 0, equals exactly −45° at ωc, and asymptotically approaches −90° as ω → ∞.

## 3.4  First-Order Low-Pass Filter

The transfer function used in this lab is:

H(s) = 1 / (s + 1) [Cutoff frequency ωc = 1 rad/s]

This function has a single real pole at s = −1 and no finite zeros. Its Bode magnitude approximation is flat at 0 dB below ωc and falls at −20 dB/decade above ωc. The exact magnitude at ωc = 1 rad/s is −3.01 dB — the half-power (−3 dB) point.

# 4.  Importance of Bode Plots in DSP

Bode plots are indispensable across a broad range of DSP and control engineering tasks:

- System Characterisation: Provides a complete picture of gain and phase behaviour across the entire frequency spectrum. Engineers can identify resonant peaks, roll-off rates, and bandwidth directly from the plot.

- Filter Design: In DSP, low-pass, high-pass, band-pass, and band-stop filter design is driven by frequency-domain specifications. The Bode plot lets the designer verify passband ripple, stopband attenuation, and transition bandwidth.

- Stability Analysis: Two key stability metrics are read directly from the Bode plot: (i) Gain Margin — amount by which gain can increase before instability; and (ii) Phase Margin — additional phase lag tolerable before instability.

- Communication Systems: Channel equalization requires precise knowledge of frequency-dependent gain and phase distortion. Bode plots characterize this distortion so that equalizer filters can compensate for it.

- Ease of Interpretation: The logarithmic frequency axis compresses a wide frequency range into a readable plot. Asymptotic straight-line approximations allow rapid hand-sketching and straightforward verification of analytical results.

# 5.  Equipment and Software

| Item | Details |
| --- | --- |
| Software | MATLAB R2018a or later (with Control System Toolbox) |
| MATLAB Functions | tf(), bode(), figure, grid on, title(), fprintf() |
| Transfer Function | H(s) = 1/(s+1) — First-Order Low-Pass Filter |
| Analysis Range | Angular frequency: 0.01 to 100 rad/s (auto, logarithmic) |
| Verification Point | ω = 1 rad/s (the −3 dB cutoff frequency) |

# 6.  Procedure

The following steps were followed to complete this laboratory experiment:

- Open MATLAB and ensure the Control System Toolbox is available (type 'ver' in the Command Window to verify).

- Create a new script file (File > New > Script) and save it as Lab4_BodePlot.m.

- Enter the complete MATLAB code provided in Section 7 (Lab Task) into the script.

- Run the script by pressing F5 or clicking the Run button in the MATLAB toolbar.

- Observe the Bode plot figure window: examine both the magnitude and phase panels carefully.

- Note the Console Output showing numerical magnitude (dB) and phase (degrees) at ω = 1 rad/s.

- Verify the output against theoretical values: −3.01 dB and −45° respectively.

- Record all observations and complete the Results section (Section 8).

# 7.  Lab Task — MATLAB Code

The MATLAB code below constitutes the complete lab task for Lab 4. It defines the transfer function H(s) = 1/(s + 1), generates the full Bode plot, and prints the magnitude and phase at the cutoff frequency ω = 1 rad/s. Students are required to type, execute, and record the output of this code during the laboratory session.

MATLAB CODE ¦ Lab Task

% ================================================================ % LAB TASK - Lab 5: Bode Plot of First-Order Low-Pass Filter % Subject : Digital Signal Processing (DSP) % Student : Rabeea Anjum % Instructor: Engr Anees ul Husnain % Dept. : Computer Systems Engineering, IUB % ================================================================ % Step 1: Define the transfer function H(s) = 1 / (s + 1) numb = [1]; % Numerator coefficients den = [1, 1]; % Denominator coefficients (s + 1) % Step 2: Create the transfer-function object H = tf(numb, den); % Step 3: Plot the Bode diagram figure; bode(H); grid on; title('Bode Plot of First-Order Low-Pass Filter'); xlabel('Frequency (rad/s)'); % Step 4: Retrieve magnitude, phase and frequency arrays [mag, phase, w] = bode(H); % Step 5: Print magnitude (dB) and phase (dig) at omega = 1 rad/s fprintf('\n--- Results at omega = 1 rad/s ---\n'); fprintf('Magnitude : %.4f dB\n', 20*log10(mag(1,1,1))); fprintf('Phase : %.4f deg\n', phase(1,1,1));

## 7.1  Line-by-Line Explanation

Each logical block of the code above is explained in the table below:

| Step / Purpose | MATLAB Statement | Explanation |
| --- | --- | --- |
| Transfer Function Definition | numb=[1]; den=[1,1]; | Defines H(s) =1/(s+1). Coefficients are row vectors for MATLAB's tf () constructor. |
| Transfer Function Object | H = tf(numb, den); | Creates a continuous-time LTI object for use with Control System Toolbox functions. |
| Bode Plot Generation | bode(H); grid on; | Plots magnitude (dB) and phase (degrees) vs. frequency (rad/s) on log-scale axes. |
| Data Retrieval | [mag,phase,w]=bode(H); | Returns magnitude, phase, and frequency arrays for numerical processing. |
| Numerical Output | fprintf(...) | Prints magnitude in dB and phase in degrees at ω=1 rad/s for verification. |

# Output:

# 8.  Results

## 8.1  Bode Plot Observations

Upon executing the MATLAB code, the Bode plot figure displayed the following characteristics for H(s) = 1/(s + 1):

- Magnitude Response: Approximately 0 dB for frequencies well below ωc = 1 rad/s, confirming the filter passes low-frequency signals without attenuation. Above ωc, magnitude decreases at −20 dB/decade, consistent with first-order roll-off.

- Phase Response: Starts at 0° for very low frequencies, decreases monotonically, and reaches exactly −45° at ωc = 1 rad/s, approaching −90° asymptotically at high frequencies.

- Cutoff Frequency: The −3 dB point is clearly identifiable on the magnitude plot at ω = 1 rad/s, verifying the theoretical prediction.

## 8.2  MATLAB Console Output

The MATLAB console output obtained at ω = 1 rad/s was:

--- Results at omega = 1 rad/s --- Magnitude : -3.0103 dB Phase : -45.0000 deg

## 8.3  Experimental vs. Theoretical Comparison

| Parameter | MATLAB Result | Theoretical Value |
| --- | --- | --- |
| Magnitude at ω = 1 rad/s | −3.0103 dB | −3.0 dB |
| Phase at ω = 1 rad/s | −45.0000° | −45.0° |
| Cutoff Frequency (ωc) | 1 rad/s | 1 rad/s |
| Roll-off Rate | −20 dB/decade | −20 dB/decade |

# 9.  Discussion

The results obtained from the MATLAB simulation are in complete agreement with the analytical predictions derived from H(s) = 1/(s+1). The magnitude of −3.01 dB and phase of −45° at ω = 1 rad/s are classical characteristics of a first-order low-pass filter and serve as benchmark values for verifying the simulation.

The Bode plot generated by MATLAB clearly shows the two key asymptotic regions: the flat 0 dB passband below ωc and the −20 dB/decade roll-off above ωc. The smooth transition, governed by the single real pole at s = −1, is a direct consequence of the first-order system dynamics.

From a practical DSP perspective, this experiment demonstrates how a simple transfer function can be rapidly analyzed using MATLAB without manual computation at each frequency. The three-output form of bode() further enhances the tool's utility in design verification workflows.

# 10.  Conclusion

This laboratory report has demonstrated the fundamental role of Bode plots in Digital Signal Processing. The key findings are:

- Bode plots provide a complete and compact frequency-domain Characterisation of any LTI system, displaying both magnitude (dB) and phase (degrees) against a logarithmic frequency axis.

- For H(s) = 1/(s+1), MATLAB confirmed the theoretical values of −3.01 dB and −45° at ωc = 1 rad/s with high precision.

- MATLAB's tf() and bode() functions offer a fast, reliable, and industry-standard workflow for constructing and analysing transfer functions in research and professional practice.

- Bode plots are essential for any engineer working in filter design, feedback control, or communication systems, as they directly expose the stability margins and frequency-shaping capabilities of a system.

Bode plots remain an irreplaceable analytical and design tool in DSP. Their intuitive graphical form, combined with the computational power of MATLAB, enables rapid system Characterisation, filter verification, and stability assessment — skills central to modern signal processing engineering.

# References

[1] Oppenheim, A. V., & Schafer, R. W. (2009). Discrete-Time Signal Processing (3rd Ed.). Prentice Hall.

[2]  Proakis, J. G., & Manolakis, D. G. (2006). Digital Signal Processing: Principles, Algorithms, and Applications (4th ed.). Prentice Hall.

[3]  MathWorks Inc. (2024). MATLAB Control System Toolbox Documentation — bode(). https://www.mathworks.com/help/control/ref/bode.html

[4]  Ogata, K. (2010). Modern Control Engineering (5th ed.). Prentice Hall.

[5]  Franklin, G. F., Powell, J. D., & Emami-Naeini, A. (2019). Feedback Control of Dynamic Systems (8th ed.). Pearson.